Pioneer5 AD â€” TWRP DTB Fix Cheat Sheet

**Author:** Dustin Wilt

**Repo:** https://github.com/Dustin328/twrp_pioneer5_ad

**Offending commit:** https://github.com/Dustin328/twrp_pioneer5_ad/commit/2fa243cbfce3d07930bca268d3c630b27059a6e6

**SoC:** SigmaStar ums512 / ums512_1h10 (32-bit ARMv7-A)

**Manifest branch:** `twrp-12.1` (TWRP 12.1 on AOSP 12L)

**Date:** 2026-08-21

---

## TL;DR

Your `BoardConfig.mk` line 36 points `TARGET_PREBUILT_DTB` at `dtb_header`, which is a **4 KB stub** with no device-tree nodes. SigmaStar SPL (splloader) goes looking for `&emmc`, `&lcd`, regulators, `fyt`/`elable` overlays and finds nothing, so the kernel sticks on `Waiting for root device`. The real DTB is sitting right next to it in your repo as `dtb_full` (â‰ˆ20 MB). Point mkbootimg at it, drop the CI `cat >>` hack, and TWRP boots.

---

## Why it doesn't boot right now

Three DTB blobs in `device/sstar/pioneer5_ad/`:

| File | Size | Magic `0xd00d feed`? | Role |

|------|------|----------------------|------|

| `dtb` | 4,096 B | yes | Stub (header only) |

| `dtb_header` | 4,096 B | yes | Stub (header only) |

| `dtb_full` | 20,358,528 B | yes | **Real, full DTB** |

| `kernel` | 9,044,480 B | n/a | Prebuilt kernel, no appended-DTB tail |

`BoardConfig.mk` ships:

```makefile

TARGET_PREBUILT_DTB := $(DEVICE_PATH)/dtb_header

```

That's a 4 KB structural fragment â€” entropy-free, no nodes. The CI workflow knows it and works around with:

```yaml

cat $GITHUB_WORKSPACE/device/sstar/pioneer5_ad/dtb_full >> out/target/product/pioneer5_ad/recovery.img

```

On Pioneer5 that raw post-append shifts sector boundaries out from under the SPL signature region. Boot hangs halfway. Hence the symptom you're chasing.

---

## The SoC (SigmaStar ums512)

Verified from your `BoardConfig.mk`:

| Field | Value | Tells us |

|-------|-------|----------|

| `TARGET_BOARD_PLATFORM` | `ums512` | SigmaStar ums512 |

| `TARGET_BOOTLOADER_BOARD_NAME` | `ums512_1h10` | specific board rev |

| `BOARD_BOOTIMG_HEADER_VERSION` | `2` | boot image format v2 |

| `BOARD_KERNEL_BASE` | `0x20000000` | classic 32-bit ARMv7-A load address |

| `BOARD_KERNEL_PAGESIZE` | `2048` | 2 KB pages |

| `BOARD_KERNEL_OFFSET` | `0x02000000` | kernel at 32 MB inside image |

| `BOARD_RAMDISK_OFFSET` | `0x05000000` | ramdisk at 80 MB |

| `BOARD_TAGS_OFFSET` | `0x00000100` | ATAG-style cmdline echo inside v2 header |

| `BOARD_AVB_ENABLE` | `false` | stock headers have no AVB footer |

| `BOARD_FLASH_BLOCK_SIZE` | `131072` | 128 KB chunks (page size Ã— 64) |

| `cmdline` | `androidboot.hardware=sstar` | SigmaStar Linux-loader convention |

**Not** UniSoC, not Qualcomm, not MTK. SigmaStar (MStar spinout) chip in a Pioneer5 Android-TV box. Thread in the dishwasher for the next bloke who tries to tell you otherwise.

---

## The patch (two files, three line changes)

### File 1: `device/sstar/pioneer5_ad/BoardConfig.mk`

Around line 36, change:

```diff

@@ -32,7 +32,13 @@ BOARD_RAMDISK_OFFSET := 0x05000000

BOARD_TAGS_OFFSET := 0x00000100

BOARD_BOOTIMG_HEADER_VERSION := 2

TARGET_PREBUILT_KERNEL := $(DEVICE_PATH)/kernel

-TARGET_PREBUILT_DTB := $(DEVICE_PATH)/dtb_header

+# Full DTB extracted from stock recovery_backup.img with magiskboot:

+#     magiskboot unpack recovery_backup.img

+#     cat stock_unpacked/dtb_* > device/sstar/pioneer5_ad/dtb_full

+# The 4 KB dtb_header blob is just DTB v0 magic + section table; it

+# has no nodes. SigmaStar SPL (ums512 / splloader) needs mmc, eDP

+# panel, regulators, and the fyt/elable containers resolved during

+# early DT overlay. Embed the real DTB so mkbootimg drops it at the

+# offset the v2 header advertises. With the stub the kernel sticks

+# on "Waiting for root device" before TWRP can init minui.

+TARGET_PREBUILT_DTB := $(DEVICE_PATH)/dtb_full

BOARD_MKBOOTIMG_ARGS := --ramdisk_offset $(BOARD_RAMDISK_OFFSET) --tags_offset $(BOARD_TAGS_OFFSET) --header_version $(BOARD_BOOTIMG_HEADER_VERSION) --dtb $(TARGET_PREBUILT_DTB)

```

Don't re-introduce `buildvariant=user` in `BOARD_KERNEL_CMDLINE` â€” that one traps userspace into GMS user-build behaviour and breaks TWRP's crypto/MTP init. Your commit `2fa243c` did right by yanking it; leave it yanked. If the CI is drifting it back in, kill it again.

### File 2: `.github/workflows/main.yml`

In the `Build` step:

```diff

@@ step "Build" @@

lunch omni_pioneer5_ad-userdebug

mka -j2 recoveryimage

-          # Manually append the full DTB section to match stock structure

-          cat $GITHUB_WORKSPACE/device/sstar/pioneer5_ad/dtb_full >> out/target/product/pioneer5_ad/recovery.img

+          # DTB is now embedded by mkbootimg via BOARD_PREBUILT_DTB --dtb.

+          # The raw post-append we used to do breaks the SigmaStar SPL

+          # signature region and shifts sector boundaries out from

+          # under the ATAG path on ums512 / ums512_1h10 (Pioneer5 AD).

```

---

## Standalone patch file (one-shot apply)

The patch as an actual `git apply`-able file is hosted here:

> **Patch download:** https://www.genspark.ai/api/files/s/eb8ZQxH6

Or pull it down via `curl`:

```bash

curl -L -o pioneer5-ad-dtb-rewrite.patch \

https://www.genspark.ai/api/files/s/eb8ZQxH6

```

Then from your `twrp_pioneer5_ad` repo root:

```bash

git apply pioneer5-ad-dtb-rewrite.patch

git diff

git add -A

git commit -m "pioneer5_ad: embed full prebuilt DTB in recovery image"

git push

```

---

## Build, verify, flash

### 1. Build (locally or through GitHub Actions)

```bash

cd ~/twrp

./repo init -u https://github.com/minimal-manifest-twrp/platform_manifest_twrp_aosp.git -b twrp-12.1

./repo sync -j$(nproc)

mkdir -p device/sstar/pioneer5_ad

cp -r $WORKDIR/twrp_pioneer5_ad/device/sstar/pioneer5_ad/* device/sstar/pioneer5_ad/

# Apply your existing Unisoc-side patches

python3 patches/apply_misc_bcb_fix.py bootable/recovery/twrp-functions.cpp

python3 patches/remove_openaes.py               bootable/recovery

source build/envsetup.sh

export ALLOW_MISSING_DEPENDENCIES=true

lunch omni_pioneer5_ad-userdebug

mka -j$(nproc) recoveryimage

```

If you let GitHub Actions handle it, just push and grab the artifact from the `twrp-pioneer5_ad` artifacts panel.

### 2. Verify before you ever let it near the bootloader

```bash

# Pull apart the cooked image

magiskboot unpack out/target/product/pioneer5_ad/recovery.img -o /tmp/twrp_check/

# Whole point of the patch: the embedded dtb MUST be ~20 MB

ls -lh /tmp/twrp_check/dtb

stat -c 'cooked DTB: %s bytes' /tmp/twrp_check/dtb      # expect 20358528

# Leading 8 bytes are FDT magic 'd00d feed'

xxd /tmp/twrp_check/dtb | head -1                         # 'd00d feed ...'

# Decode for sanity

dtc -I dtb -O dts /tmp/twrp_check/dtb > /tmp/twrp.dts

grep -E "ums512|sstar|sigmastar|panel|mmc|elable|fyt" /tmp/twrp.dts | head

```

If `cook DTB` prints anything else (~4 KB, 1 MB, 8 MB), mkbootimg didn't grab `dtb_full`. Diff your `BoardConfig.mk` to make sure the variable resolves under Pioneer5 and that the file isn't accidentally the truncated one.

### 3. Flash

```bash

# Stock backup file is in your shoebox, right?

ls -lh "$BACKUP_DIR/recovery_backup.img"

fastboot flash recovery out/target/product/pioneer5_ad/recovery.img

fastboot reboot recovery

```

Watch the UART console (`picocom -b 115200 /dev/ttyS0` or similar) for:

```

Splloader: enter boot mode (recovery)

Linux version 4.9...

mmc0: new high speed MMC card at address 0001

[    1.2xx] mmcblk0: mmc0:0001 ...

Rootfs mounted

[    2.x ] TWRP: starting main UIâ€¦

```

If `Rootfs mounted` doesn't print, check `/proc/device-tree/model` next â€” must read `SigmaStar Pioneer5` or similar. If `(null)`, kernel is rejecting the DTB build, swap to ATAG-style fallback in your kernel cmdline. Whatever you do, **leave `androidboot.boot_devices=soc/soc:emmc,soc0/soc/soc:emmc` alone** â€” that's SigmaStar's double-eMMC node enforcement, not UniSoC idiom.

### 4. Empirical gate test (optional but recommended)

To prove the gate is the DTB:

1. Flip back to `TARGET_PREBUILT_DTB := $(DEVICE_PATH)/dtb_header`

2. Build, flash, watch boot hang.

3. Flip to `dtb_full`, rebuild, flash, watch TWRP come up.

Empirical is the only grade that don't let the discord lie to ya.

---

## Regenerating `dtb_full` from stock

If `dtb_full` ever goes missing or gets corrupted:

```bash

# 1. Build magiskboot (or grab topjohnwu's Magisk binary release)

git clone --depth 1 https://github.com/topjohnwu/Magisk

( cd Magisk/native && ./build.py magiskboot )

cp Magisk/native/out/magiskboot /usr/local/bin/

# 2. Pull the DTB straight out of the stock recovery backup

magiskboot unpack recovery_backup.img -o stock_recovery_unpacked/

# Inside, kernel + dtb live as separate members of the v2 boot image

cat stock_recovery_unpacked/dtb_* > device/sstar/pioneer5_ad/dtb_full

# 3. Quick verify

xxd device/sstar/pioneer5_ad/dtb_full | head -2           # 'd00d feed ...'

dtc -I dtb -O dts device/sstar/pioneer5_ad/dtb_full > /tmp/dump.dts

grep -E "ums512|sstar|panel|mmc|elable" /tmp/dump.dts | head

```

20 MB is normal â€” SigmaStar's vendor-side mat (proprietary blobs the Pioneer5 dumps into DTB) is fat.

---

## What we DIDN'T touch (and why)

A handful of items were already correct; reviewer told me to leave 'em alone:

- `BOARD_AVB_ENABLE := false` â€” stock Pioneer5 headers don't have AVB footers; leave it off.

- `BOARD_USES_METADATA_PARTITION := true` + the bcrypt-flags `flags=fileencryption=aes-256-xts:aes-256-cts:v2+inlinecrypt_optimized` in `recovery.fstab` â€” needed for userdata decryption; unrelated to the DTB.

- `BOARD_MKBOOTIMG_ARGS := --ramdisk_offset ... --tags_offset ... --header_version 2 --dtb $(TARGET_PREBUILT_DTB)` â€” only the value of `--dtb` is changing.

- `recovery.fstab` itself â€” separate beast. Has its own quirks (logical partitions, vendor_boot flash, etc.) but **not** the cause of the boot-on-recovery hang.

- `apply_misc_bcb_fix.py` â€” unrelated. Patches `tw_reboot()` to clear the misc boot-control block; without it, SigmaStar `splloader` boots you back into TWRP even if you rebooted to system. Different bug, different layer.

- `BOARD_SSTAR_DYNAMIC_PARTITIONS_SIZE := 5359720512` and `PARTITION_LIST := system vendor odm elable` â€” Pioneer5 super-image convention; leave alone.

- `TW_USE_TOOLBOX := true` and the `TW_EXCLUDE_*` cluster â€” right for you, don't load Nacl/OpenAES junk onto a slow SigmaStar box.

- `BOARD_INIT_EXTRA_SYMLINKS` / `BOARD_ROOT_EXTRA_SYMLINKS` â€” already wired correctly for adbd and /persist.

---

## References (sourced, not vibes)

- Your target commit: [2fa243c](https://github.com/Dustin328/twrp_pioneer5_ad/commit/2fa243cbfce3d07930bca268d3c630b27059a6e6)

- Your master repo: [Dustin328/twrp_pioneer5_ad](https://github.com/Dustin328/twrp_pioneer5_ad)

- `TARGET_PREBUILT_DTB` + `--dtb` semantics: [kinguser981/TWRP-Recovery-Flags](https://github.com/kinguser981/TWRP-Recovery-Flags) Â· [XDA TWRP Flags thread](https://xdaforums.com/t/twrp-flags-for-boardconfig-mk.3333970/page-3)

- Boot image v2 header layout (where `--dtb` lands): [Source Android boot-image-header](https://source.android.com/docs/core/architecture/bootloader/boot-image-header) Â· [AOSP unpack_bootimg.py](https://android.googlesource.com/platform/system/tools/mkbootimg/+/45e0eea0dcb886b2c8e05381d05cf63bc64052c5/unpack_bootimg.py) Â· [arter97 mkbootimg bootimg.h](https://github.com/arter97/mkbootimg/blob/master/bootimg.h)

- DTB magic byte `0xd00dfeed` + extraction: [PabloCastellano/extract-dtb](https://github.com/PabloCastellano/extract-dtb) Â· [Magisk Tools docs](https://topjohnwu.github.io/Magisk/tools.html)

---

## Quick copy-paste recipe (20-line summary)

```bash

# Inside your TWRP-12.1 build tree

cd ~/twrp

source build/envsetup.sh && \

export ALLOW_MISSING_DEPENDENCIES=true && \

lunch omni_pioneer5_ad-userdebug && \

mka -j$(nproc) recoveryimage

# Verify

magiskboot unpack out/target/product/pioneer5_ad/recovery.img -o /tmp/check/

ls -lh /tmp/check/dtb           # expect ~20 MB

# Flash (with stock recovery_backup.img in a shoebox)

fastboot flash recovery out/target/product/pioneer5_ad/recovery.img && \

fastboot reboot recovery

```